# Lab 06 – Worksheet

| Name: Mohammad Husain Naqvi | ID: 11062 | Section: T-4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Block Diagrams

Comments

## Task 2

Modules + Testbench:

TOP Module: \ `timescale 1ns / 1ps //////////////////////////////////////////////////////////////////////////////////// //// Company: //// Engineer: //// //// Create Date: 08.10.2026 13:36:04 //// Design Name: //// Module Name: ALUtop //// Project Name: //// Target Devices: //// Tool Versions: //// Description: A //// //// Dependencies: //// //// Revision: //// Revision 0.01 - File Created //// Additional Comments: //// //////////////////////////////////////////////////////////////////////////////////// module top_alu32bit_fpga ( input wire clk, input wire btnC, input wire [15:0] sw, output wire [15:0] led ); wire rst_db; debouncer db_rst ( .clk (clk), .pbin (btnC), .pbout (rst_db) ); wire [31:0] sw_readData; wire [15:0] sw_led_mirror; switches sw_if ( .clk (clk), .rst (rst_db), .writeData (32'b0), .writeEnable (1'b0), .readEnable (1'b1), .memAddress (30'b0), .sw (sw), .readData (sw_readData), .leds (sw_led_mirror) ); // Form 4-bit ALUControl: sw[2:0] mapped with MSB = 0 wire [3:0] op = {1'b0, sw_readData[2:0]}; // Procedure Step (e): Fixed 32-bit operands wire [31:0] A = 32'h10101010; wire [31:0] B = 32'h01010101; wire [31:0] result; wire zero; // ALU instantiation with 4-bit control ALU_32bit alu ( .A (A), .B (B), .ALUControl (op), .Result (result), .Zero (zero) ); // Drive 16 board LEDs assign led = result[15:0]; endmodule ALU : `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 08.10.2026 02:54:14 // Design Name: // Module Name: acsd // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module ALU_32bit ( input wire [31:0] A, input wire [31:0] B, input wire [3:0] ALUControl, // 4-bit ALU control output reg [31:0] Result, output wire Zero ); always @(*) begin case (ALUControl) 4'b0000: Result = A & B; // AND (Switches: None) -> 0x0000 4'b0001: Result = A ¦ B; // OR (Switches: 0) -> 0x1111 4'b0010: Result = A + B; // ADD (Switches: 1) -> 0x1111 4'b0011: Result = A ^ B; // XOR (Switches: 0, 1) -> 0x1111 4'b0100: Result = A << B[4:0]; // SLL (Switches: 2) -> 0x2020 4'b0101: Result = A >> B[4:0]; // SRL (Switches: 0, 2) -> 0x0808 4'b0110: Result = A - B; // SUB (Switches: 1, 2) -> 0x0F0F 4'b0111: Result = A - B; // BEQ (Switches: 0, 1, 2) -> 0x0F0F default: Result = 32'b0; endcase end assign Zero = (Result == 32'b0); endmodule Debouncer: `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: Muddassir Ali // Module Name: debouncer // Project Name: Counter // Target Devices: Baasys 3 // ////////////////////////////////////////////////////////////////////////////////// module debouncer ( input wire clk, input wire pbin, output reg pbout ); reg [19:0] counter = 20'b0; reg sync_0 = 1'b0; reg sync_1 = 1'b0; // Double-flop synchronizer always @(posedge clk) begin sync_0 <= pbin; sync_1 <= sync_0; end // Counter filter (~10ms at 100MHz) always @(posedge clk) begin if (sync_1 == pbout) begin counter <= 20'b0; end else begin counter <= counter + 1'b1; if (counter >= 20'd1_000_000) begin pbout <= sync_1; counter <= 20'b0; end end end endmodule SWITHCES: `timescale 1ns / 1ps module switches ( input wire clk, input wire rst, input wire [31:0] writeData, input wire writeEnable, input wire readEnable, input wire [29:0] memAddress, input wire [15:0] sw, output reg [31:0] readData, output reg [15:0] leds ); always @(posedge clk or posedge rst) begin if (rst) begin readData <= 32'b0; leds <= 16'b0; end else begin if (readEnable) begin readData <= {16'b0, sw}; end if (writeEnable) begin leds <= writeData[15:0]; end end end endmodule TESTBENCH: `timescale 1ns / 1ps module ALU_tb; reg [31:0] A; reg [31:0] B; reg [3:0] ALUControl; wire [31:0] Result; wire Zero; ALU_32bit uut ( .A (A), .B (B), .ALUControl (ALUControl), .Result (Result), .Zero (Zero) ); initial begin // Fixed operands per step (e) A = 32'h10101010; B = 32'h01010101; ALUControl = 4'b0000; #50; // 1. AND (0000) -> 0x00000000 ALUControl = 4'b0000; #50; // 2. OR (0001) -> 0x11111111 ALUControl = 4'b0001; #50; // 3. ADD (0010) -> 0x11111111 ALUControl = 4'b0010; #50; // 4. XOR (0011) -> 0x11111111 ALUControl = 4'b0011; #50; // 5. SLL (0100) -> 0x20202020 ALUControl = 4'b0100; #50; // 6. SRL (0101) -> 0x08080808 ALUControl = 4'b0101; #50; // 7. SUB (0110) -> 0x0F0F0F0F ALUControl = 4'b0110; #50; // 8. BEQ (0111) -> 0x0F0F0F0F ALUControl = 4'b0111; #50; // 9. Zero Flag Check (Equal inputs) A = 32'hA5A5A5A5; B = 32'hA5A5A5A5; ALUControl = 4'b0110; // SUB -> Result = 0, Zero = 1 #50; $finish; end endmodule

Results:

## Task 3

## RTL Schematics

The simulation tests all ALU operations using fixed inputs A = 0x10101010 (269488144) B = 0x01010101 (16843009). AND: ( 0 – 100ns ) 1. AND (ALUControl = 0): Result = 0 (0x00000000). All bits are 0, so Zero flag = OR: ( 100ns – 170 ns) 2. OR (ALUControl = 1): Result = 286331153 (0x11111111), Zero flag = 0. ADD: (170 -200ns) ADD (ALUControl = 2): Result = 286331153 (0x11111111), Zero flag = 0. XOR: ( 200 – 270 ns ) XOR (ALUControl = 3): Result = 286331153 (0x11111111), Zero flag = 0. SLL: ( 270 – 310 )ns SLL (ALUControl = 4): Shifts A left by B[4:0] (1 bit). Result = 538976288 (0x20202020). SRL ( 310 – 370 ) SRL (ALUControl = 5): Shifts A right by B[4:0] (1 bit). Result = 134744072 (0x08080808). SUB: ( 370 – 420 )ns SUB (ALUControl = 6): A - B gives Result = 252645135 (0x0F0F0F0F). BEQ ( <440 ) BEQ (ALUControl = 7): Executes subtraction datapath; Result = 252645135 (0x0F0F0F0F).

GithubLink:

https://github.com/MohammadNaqvi1098/CALabs-Git-mn11062.git

# Lab 05 – Designing FSM Using FPGA Switches and LEDs

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Task 1 | - | - | /20 | /20 |
| Task 2 | /20 | /15 |  |  |
| Task 3 | /10 | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |